
[image: image7.png]



Registered Charity No 223015

Cononley Village Institute

Main Street 

Cononley BD20 8NT

Risk Assessments

This document contains Fire Risk Assessment

Fire Evacuation Plan

General Building Risk Assessment.

Please familiarise yourself with these documents and procedures.

Main points to note:

1. That all hirers must have a personal evacuation plan for vulnerable people

2. That regular hirers must visually check fire alarm system is operating when entering the building

3. That all hirers must be aware of the building capacity. Capacity varies depending on seating arrangements

4. It is suggested that large events nominate a fire marshal who is aware of emergency plan
Cononley Village Institute Advisory Committee have used a series of fire safety criteria to set the following maximum occupancy levels based on knowledge of how the building is used, its detailed layout and the multiple ways in which this flexible space can be set out. Some events will have part dining style and part standing room, for example, which will lead to slightly increased or decreased capacity. All hirers are expected to use common sense principles when determining the number of people using the building, ensuring fire exits are clear and there is enough space between tables for dining events. 
	
	Main hall
	Meeting room
	Whole building

	Standing only and assembly hall/dance floors/concerts
	120
	40
	160



	Dining style
	75
	33
	108



	Theatre style seated arrangement 
	100 (NB if stage, projector or large performance area recommended  audience 70)
	n/a
	n/a




Introduction


Fire Safety Risk Assessment

This form is intended to assist small businesses in carrying out the fire safety risk safety assessment. It is suitable for use in simple premises, with a simple internal layout and small numbers of people present, e.g. small offices, shops or industrial units and where people can escape unaided. In buildings with complicated escape arrangements, large numbers of people or high fire safety risk processes a more comprehensive fire safety risk assessment may be necessary.

The enclosed information follows a simple 5 step guide to completing a suitable assessment. The fire safety risk assessment does not need to be complicated, it should be a common sense review to identify the hazards (what could start a fire and what could burn), followed by consideration of the possible effects of a fire on people using the building. The important thing is that the fire safety risk assessment is systematic to ensure that every part of the premises is assessed. Every room, space or area, especially those not often used, should be included.

If you identify any significant risks you should consider if they can be reduced: first by removing the hazards and secondly by providing fire protection measures (e.g. automatic fire detection). These points should be addressed within the ‘What needs to be done to make each situation safe?’ section. If your premises are located within a larger building and if a fire in your business could affect your neighbours, you should share the findings of your fire safety risk assessment with your neighbours.

Your fire safety risk assessment must be kept up to date. It is important to update the assessment if anything is changed that might affect the risk (e.g. new ignition sources or use of flammable liquids etc.). If you identify that either the likelihood of a fire occurring or there is a risk to people you may want to consider seeking further assistance.


Emergency Plan

The findings of the fire safety risk assessment should be used to prepare an emergency plan. The plan should set out to include what to do in case of fire, identify the escape routes, where to assemble and how to contact the Fire and Rescue Service. The fire safety equipment and any equipment that provides a possible ignition source (e.g. cooking, electrical appliances, etc.) should be regularly maintained to reduce the chance of fire and the risk to people. Frequent checks should also be made to make sure that the storage of materials, especially flammable materials, does not increase the risk of fire or prevent the use of escape routes. It is important that all members of staff know how to prevent fires and what to do if a fire does occur. To make sure that the emergency plan works regular fire drills should be carried out. For further guidance on the Regulatory Reform (Fire Safety) Order 2005 and for information and guidance on carrying out your fire safety risk assessment visit https://www.gov.uk/government/collections/fire-safety-law-and-guidance-documents-for-business
and follow the links in fire safety.

The 5 Step Guide to Fire Safety Risk Assessment

1. Identify the fire risks

2. Identify people at risk

3. Evaluate, remove, reduce and protect from risk

4. Record, plan, inform, instruct and train

5. Review
	FIRE SAFETY RISK ASSESSMENT

	
	

	DETAILS

	
	

	Company Name:
	Cononley Village Institute (Charity)

	
	

	Premises Name:
	Cononley Village Institute

	
	

	Address:
	Main Street

Cononley

North Yorkshire

BD20 8NT

	
	

	
	

	
	

	
	

	
	

	Responsible Person:
	Becky Ward

	
	

	Position:
	Secretary of Cononley Village Institute Committee

	
	

	Date of Assessment:
	 29//7/25 agreed at CVIAC on 15/9/25

	
	

	Carried out by:
	 Becky Ward and Mark Allum

	
	

	Position:
	Secretary Cononley Village Institute Committee and CPC Rep

	
	

	Use of the Premises:
	Community Building used for ad hoc community events, weekly classes, private hire and a Day Nursery during term-time

	
	

	
	

	
	

	
	

	
	

	OCCUPANCY

	
	

	Times in Use:
	Weekdays:
	07.00-22.00
	Weekends:
	09.00-23.30

	
	

	Total Numbers of Staff:
	Weekdays:
	1 part-time
	Weekends:
	1 part-time

	
	

	Total of all Persons Present:
	Weekdays:
	Approx. 30
	Weekends:
	15-100

	
	

	SIZE

	
	

	Total Size of Premises (M2):
	153.5 M2

	
	

	Number of Storeys:
	1 plus loft storage

	
	

	Number of Basements:
	1 limited access


INDEMNITY

This document has been produced as a tool to assist you in completing a fire safety risk assessment of your premises. It is used entirely at your own risk to identify what you consider are your significant findings, and also whether you consider the information therein to be suitable and sufficient. It is in no way exhaustive and West Yorkshire Fire and Rescue Service accepts no liability whatsoever for any circumstances which may arise as a result of using this tool. 
	Step 1 - Identify Fire Hazards (Sources of Ignition)

	
	
	
	

	Type
	Location
	Are existing control measures suitable?

	Naked Flames
	Gas cooker – annual service
Gas boiler – annual service
Fire extinguishers – annual check

Monthly fire extinguisher check 
	
	YES
	
	
	

	Portable Heaters and Heating Equipment 
	AC / heating unit (kitchen) subject to 5 yearly electrical test
Gas central heating – confirmed with LPS that radiators are not subject to gas safety checks.
	
	YES
	
	
	

	Electrical Equipment
	Various items (eg kettles) which are permanently in building are PAT tested annually.

Ceiling fan (meeting room) subject to 5 yearly electrical test

CVI users will have smart phones, laptops, tablets etc containing batteries. 

Solar battery – well positioned in ventilated area

Chair lift battery – in a well ventilated space but in a fire exit route but directly below a fire detector
Some hirers bring non PAT tested electrical equipment into the building – hire conditions amended to say ‘must be in good condition, used safely and not left unattended if plugged in.’

6 monthly fire alarm service

Monthly fire alarm test

Monthly electrical ‘flick’ test

Annual test emergency lighting
	
	YES
	
	
	

	Cooking Equipment
	Gas Hob, Electric Double Oven, Refrigerator x 2, Water Heater, Dishwasher

	
	YES
	
	
	

	Work Process Risk(s)
	Use of kitchen only. Clear guidelines for use
	
	YES
	
	
	

	Arson
	Risks very low in village

Smoke and Heat Alarms within the full alarm system
	
	YES
	
	
	

	Smoking Materials
	No smoking allowed.

No candles permitted

Smoke and Heat Alarms with full alarm system. 
Smoke detector in loft


	
	YES
	
	NO
	

	Other Sources

(including Contractors)
	Contractors – Clock maintenance, Plumber, Heating Engineer, Electrician, Decorators.

Nursery undertakes own Risk Assessment


	
	YES
	
	
	

	
	
	
	
	
	
	

	
	If you have answered NO to any question above complete the details below: -
	

	
	What needs to be done to make each situation safe?
	Action required by whom
	Date

due
	Date

complete
	

	
	· Smoke detector in loft is not audible when doors are closed and fire safety check identified need for hard wired alarm

	Discussion at CVIAC at next meeting September 2025
	Decision before March 2026
	
	

	
	· No detector or alarm in store room
	Discussion at CVIAC at next meeting September 2025
	Decision before March 2026
	
	

	
	· No fire detector or alarm in other loft area (above back room and entrance hall, but each room has a fire alarm sensor
	Discussion at CVIAC at next meeting September 2025
	Decision before March 2026
	
	

	
	
	
	
	
	

	
	
	
	
	
	
	


	Step 1 – Identify Fire Hazards (Sources of Fuel and Oxygen)

	
	
	
	

	Type
	Location
	Are existing control measures suitable?

	Wood / Paper /Cardboard
	Wooden floors and cupboards

General use paper in building (eg nursery printer paper)

Bulk items (eg toilet roll) kept in loft store area.

Carboard boxes for storage
	
	YES
	
	
	

	Plastics / Rubber / Foam 
	Minimal
	
	YES
	
	
	

	Retail Stock 
	None
	
	YES
	
	
	

	Furniture and Fixings 

(curtains, blinds etc)
	Curtains and blinds fireproofed.

Furniture standard village hall.

All stored correctly
	
	YES
	
	
	

	Flammable Material

(gases / liquids / paints / thinners / glues)
	Some wine / beer on site for events

Small amounts of domestic type cleaning materials in building (primarily kitchen and stored in the loft area) 

Tins of paint and small quantities white spirits generally stored in the loft area 

	
	YES
	
	
	

	Display Materials or Decorations
	Some display materials on noticeboards. 

Christmas decorations stored in loft. When on display keep away from potential sources of ignition.
	
	YES
	
	
	

	Waste Materials

(refuse, packaging)
	Disposed of to outside bins as they arrive. 
	
	YES
	
	
	

	Building Structure
	Stone/Brick building.  

Refurbished and rewired 2012.

Clock Tower refurbished 2020.

Insulation and major building improvements 2024. 
	
	YES
	
	
	

	Oxygen Supplies
(air conditioning units / bottles / piped supply)
	None
	
	YES
	
	
	

	

	
	If you have answered NO to any question above complete the details below: -
	

	
	What needs to be done to make each situation safe?
	Action required by whom
	Date

due
	Date

complete
	

	
	· 
	
	
	
	

	
	· 
	
	
	
	

	
	· 
	
	
	
	


	Step 2 – Identifying People at Risk

	


	Type
	Findings

	a) Sensory Risk:
People with visual and / or hearing impairment(s)
	Very loud and visual alarms. 
People with impairment not at risk.


	b) Mobility Risk:
People with physical impairments
	Disabled access available 
Exit from Hall would need some assistance with chair lift or stairs 
Young children would need assistance to exit. 
We do not have evac chair as not trained.  

	c) Familiarity Risk:
People who may be new to the premises and not familiar to its layout, seasonal workers, contractors, visitors or customers
	Building small and simple to understand - 3 main rooms. Main entrance and fire exit (Main Hall) are well signed. Fire plan / layout attached to risk assessment and on display throughout the building. Fire alert notices on display
Most users are local and familiar with it. Regular hirers reminded annually about evacuation procedure.

Where to find the information for private hirers is available in the hire documentation and those unfamiliar with building are generally given an induction by caretaker



	d) Numbers Risk:

Large numbers of people, small numbers of disabled people 
	Some events attract large numbers, eg Beer Festival.  Some are quite lively, individual organisers responsible for carrying out risk assessments at large events and recommended that they nominate a fire marshal
Hirers are aware of building capacity

	e) Lone  Workers / People Working in Isolation / Others
	No lone-worker normally other than Caretaker, relief caretaker or CVI member.    All are familiar both with the Hall and the fire risks.  

(See Lone Worker Policy)

	

	Additional Information:
To ensure that all electrical and fire tests schedules meet current requirements


	
	Floor area
	Use

	
	Main Hall
	Public use / classes /meetings / nursery

	Step 1 – Identify Fire Hazards

	Sources of ignition
	Sources of fuel
	Sources of oxygen

	Main Hall:

Fuse box

Solar battery

Christmas lights


	Cushioned chairs
Curtains

Blinds

Wooden cupboards

Insulation
	No additional sources

	Entrance Hall and WC:

Chair lift battery


	Nursery papers

Coats in corridor
	No additional sources

	Kitchen:

Gas hob

Gas boiler

Electric oven

Dishwasher

Fridge

Kettle

Microwave

AC unit

Fuse box
	Gas 
	No additional sources but hatch is unlikely to be fire door standard

	Meeting room:

Fridge

Ceiling fan


	Toys

Books and paper

Curtains

Blind
Children’s paint (minimal fire risk)
	No additional sources

	Store room and loft:


	Flammable liquids

Paper

Toys
	No additional sources

	Basement
	None – clear area
	No additional sources

	Step 2 – People at risk
	
	

	All users – specific concern older / disabled people and children


	Step 3 – Evaluate, remove, reduce and protect from risk

	(3.1) Evaluate the risk of the fire occurring

(3.2) Evaluate the risk to people from a fire starting in the premises

(3.3) Remove and reduce the hazards that may cause a fire

(3.4) Remove and reduce the risks to people from a fire


	Low – annual PAT test of electrical equipment, 5 yearly electrical check, solar panel well ventilated

Non smoking building.

No naked lights, fan heaters or flames (candles, griddles, indoor BBQ’s)

Only significant fuel source is gas in the kitchen. Possible accidental fire in kitchen 

Low – Small building with easy layout. 2 exits with doors 1100,mm wide (220 capacity) which is above building capacity. Main concern would be fire starting in the kitchen. Both kitchen doors are fire doors with automatic closure. Fire detector located in kitchen.
Curtains, blinds and furnishings are fire retardant
Ensure private hirers are aware of procedures and responsibility of fire risk assessments and emergency plan

Emergency Evaculation Plan is on display

Fire alert notice on display
Recommended that large events have fire marshal
Personal Emergency Evaculation Plan (PEEP) needs to be written for general use. Large events / groups with vulnerable individuals should have individual PEEP
Store / loft area is restricted access to those familiar with building as exit is via a second room




	Step 3 ~ Evaluate, Remove, Reduce and Protect From Risk

	
	
	
	

	Are ignition sources controlled to reduce the chances of fire?
	YES
	
	
	

	Are combustible materials kept away from ignition sources?
	YES
	
	
	

	Are all windows and openings closed last thing at night?
	YES
	
	
	

	Is your fire alarm system adequate for your premises? 
	YES
	
	NO
	

	Will everybody be warned if the fire alarm operates?
	YES
	
	
	

	Can everyone escape without assistance?
	
	
	NO
	

	Is the means of escape adequate for the layout of the building?
	YES
	
	
	

	Are all fire exits easily identified by correct signage?
	YES
	
	
	

	Are escape routes free from obstruction and storage?
	YES
	
	
	

	Are all doors on escape routes easily opened without a key?
	YES
	
	
	

	Do all doors on escape routes open in the direction of escape?
	YES
	
	NO
	

	Can everyone escape in a reasonable time?
	YES
	
	
	

	Are escape routes adequately lit if the regular lighting system fails?
	YES
	
	
	

	Is the lighting adequate to illuminate circulation routes?
	YES
	
	
	

	Do you have fire fighting equipment?
	YES
	
	
	

	Is the fire fighting equipment adequate for the risks present?
	YES
	
	
	

	Are housekeeping and general waste management adequate?
	YES
	
	
	

	Are security arrangements sufficient to prevent unauthorised access?
	YES
	
	
	

	Are measures adequate to prevent the incidents of arson?
	YES
	
	
	

	Are your premises free of any large open roof spaces or concealed ceiling voids?
	
	
	NO
	

	Would a fire in your premises be contained wholly within it?
	YES
	
	
	

	Can the fire service easily get to your premises?
	YES
	
	[image: image8.png]N, K
A

F|re
assembly
point





	

	
	
	
	
	
	
	

	
	If you have answered NO to any question above complete the details below: -
	

	
	What needs to be done to make each situation safe?
	Action required by whom
	Date

due
	Date

complete
	

	
	· Disabled evacuation from main hall
	An organiser must be aware of any disabled person requiring help to exit the building in case of fire and provide relevant assistance (personal emergency evacuation plan)
	N/A - ongoing
	
	

	
	· Young children evacuation from building. 
	Any event organiser must take responsibility for the safe evacuation of children.  Stepping Stones Nursery have their own fire risk assessment / evacuation procedure
	N/A - ongoing
	
	

	
	· Fire Safety check 10/4/25 recommended new batteries in unit (completed), replacing fused spur feeding (completed), hall door closure connected to fire alarm (completed), relocate sounder beacon in main hall (completed). Further recommendations included installation of 3 smoke detectors in loft with remote indicator, smoke detector in store room, meeting room, cloakroom area, at high level in main hall (7 in total). Also recommended a sounder beacon in accessible toilet


	Discussion at CVIAC at next meeting September 2025
	Decision before March 2026
	
	

	
	· Exit from meeting room – door does not open outwards but exit easily identifiable
	NFA – regular hirers reminded of this
	
	16/9/25
	

	
	· 
	
	
	
	

	
	· 
	
	
	
	

	
	
	
	
	
	
	

	Step 4 ~ Record, Plan, Inform, Instruct and Train

	
	
	
	

	You must record your fire safety arrangements – this includes:

	
	
	
	

	Have you made an emergency plan?
	YES
	
	
	

	Have you provided fire instruction and staff training?
	YES
	
	
	

	Are there records of fire drills to test your training and emergency plan?
	YES
	
	
	

	Are there records of maintenance on all fire safety measures?
	YES
	
	
	

	Have you recorded the significant findings of this assessment on Page 9?
	YES
	
	
	

	
	
	
	
	
	
	

	
	If you have answered NO to any question above complete the details below: -
	

	
	What needs to be done to make each situation safe?
	Action required by whom
	Date

due
	Date

complete
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	

	Step 5 ~ Review
	
	

	
	
	
	

	Your fire safety risk assessment must be kept up to date
	

	
	
	
	
	
	
	

	Date of next review

It is recommended that you review your fire safety risk assessment regularly (recommended every 12 months) OR if you make changes to the layout of your premises, any changes to work processes, significantly increase the amount of combustible materials stored or displayed or sources of ignition, change your opening hours (e.g. to include night time opening etc.) or any failures in your fire safety precautions then you should review your fire safety risk assessment.  
	1st November 2026
	

	
	
	


For further information visit the website – www.firesafetyguides.communities.gov.uk or contact the Fire Protection Section of West Yorkshire Fire & Rescue Service 

on 01274 682311.
A copy of this form can be downloaded from www.westyorksfire.gov.uk and follow the links from fire safety then legislative fire safety.

SIMPLE PREMISES FIRE SAFETY RISK ASSESSMENT

LEVEL OF FIRE RISK
In premises where there is a likelihood of a fire starting and spreading quickly, or a fire could start and grow undetected, and affecting the escape routes before people can use them, then the level of risk should normally be regarded at ‘higher’. Such premises might include those where significant quantities of flammable materials are used or stored; ready sources of ignition are present, e.g. heat producing machinery and processes; premises where significant numbers of the people are present and might move slowly or be unable to move without assistance; and premises where the construction provides hidden voids or flues through which a fire could quickly spread.

In premises where there is a low occupancy level and all the occupants are able bodied and capable of using the means of escape without assistance; very little chance of a fire starting; few if any highly combustible or flammable materials or other fuels for a fire; fire is unlikely to spread quickly; and will be quickly detected so that all people will quickly know that a fire has occurred and can make their escape, then the risk can usually be regarded as ‘lower’.

In most cases however, the risk will usually be ‘normal’.

Taking into account both the active and passive fire prevention measures and general fire precautions observed at the time of this fire safety risk assessment, it is considered that the hazard from fire (likelihood of fire) at these premises is:

	Low
	
	
	Normal
	x
	
	High
	


In this context, a definition of the above terms is as follows: -

Low:

Unusually low likelihood of fire as a result of negligible potential sources of ignition.

Normal:
Normal fire hazards (e.g. potential ignition sources) for this type of occupancy, with fire hazards generally subject to appropriate controls.
High:
Lack of adequate controls applied to one or more significant fire hazards, such as to result in significant increase in the likelihood of fire.
Note that, although the purpose of the above is to place the risk fire in context, the approach to fire safety risk assessment is subjective and for guidance only. All hazards and deficiencies identified in this fire safety risk assessment should be addressed by implementing all the recommendations contained in the following action plan. 

The fire risk assessment should be reviewed regularly (recommended to be annually).
SIMPLE PREMISES
FIRE SAFETY ACTION PLAN

	Each item should be  allocated a priority as follows: -

	Priority  1:
	Needs attention immediately

	Priority  2:
	Needs attention within 1 month

	Priority  3:
	Needs attention within the next 6 months               

	Priority  4:
	Needs attention within the next 12 months


	Ref:
	Action

Required
	Priority
	Action by

Whom
	Completion

Date

	1
	To discuss if further fire detectors are needed
	3
	CVIAC
	March 2026

	2
	Carry out or observe a (nursery) fire drill
	3
	CVIAC
	March 2026

	3
	Check hatch fire resistance (kitchen)
	2
	BW
	10/1/25 – probably plywood so not fire resistant. Asked for quote to replace but not provided. To discuss at CVIAC 15/9/25

	4
	To decide if need an independent fire risk assessment in 2026
	4
	CVIAC
	15/9/25 Agreed not required as fire risk assessment is comprehensive

	
	
	
	

	Page
	
	of
	


CONONLEY VILLAGE INSTITUTE FIRE EVACUATION  PLAN 
FIRE  ESCAPE PROCEDURE - In the event of a fire please assemble outside Shackleton Ghyll Farm on the other side of the beck. It is the responsibility of the hirer to contact the fire brigade.  There is NO telephone in the building. Mobile reception is best on Main Street. The Wifi code is DrYx3k3nu3qpFn
[image: image9.png]


[image: image10.png]









​












PATH to / from Main Street
[image: image11.png]


[image: image12.png]






     
[image: image1]
                                                                              
[image: image2]



[image: image3]










                                                                         
[image: image4]



[image: image5]Track to Shackleton Ghyll Farm Assembly Point



	
	Fire extinguisher
	
	Fire blanket

	
	Emergency light
	
	Fuse box (in cupboard under fire extinguisher in Main Hall and high on wall behind kitchen door)

	
	Emergency Exit light
	
	Fire Alarm break glass

	
	First aid kit
	
	Smoke detector

	
	Smoke Alarms
	
	Fire Alarm Panel

	 SHAPE  \* MERGEFORMAT 



	Fire Marshal High Viz
	
	


Cononley Village Institute

ANNUAL GENERAL RISK ASSESSMENT – October 2024
ACTION PLAN

	GENERAL – all areas (includes Entrance, Hall, Kitchen, Utility Room, Back Room, Storeroom and Toilet areas)
	Observation
	LEVEL OF RISK
	PERSON(S) RESPONSIBLE
	 ACTION REQUIRED

	1. Annual Fire Risk Assessment


	Annual Fire Risk Assessment

(see separate document).

Annual Fire Alarm Equipment Testing and service in place
(Minimum) Monthly Fire Alarm tests, emergency lighting ‘flick test’ and fire extinguisher checks undertaken and dates recorded.
	4
	CVI Committee

B Ward

CVI Committee

B Ward

K Hardy

CVI Caretaker


	Fire Risk Assessment completed July 2025, for discussion at CVIAC 15/9/25 and agree if need an independent one in 2026. Agreed not required as fire risk assessment is comprehensive


	2. Electrical and Gas Safety Certificates Record and

Boiler Service Record


	CVI / building equipment PAT tested completed on an annual basis.

Private hires cannot always adhere to using PAT tested electrical equipment. Hire conditions state that electrical ‘must be in good condition, used safely and not left unattended if plugged in’.
Full electrical check every 5 years – last done 2025, next due 2030

Boiler serviced on an annual basis.

Fire procedures in place should an incident occur
	4
	CVI Committee
	Adhere to testing programme. 

	3. Access path
	Risk of slipping in winter if snow / ice
	2
	K Hardy
	Ensure grit box (on decking) is stocked in winter. Ensure CVI snow shovel is available to use on path. 

	4. Access path
	Risk of slipping in dark
	2
	All
	External lighting is available – timer system when leaving the building

	5. Fire Exit - Main Hall
	Potential hazard exiting on to a busy road.


	3
	CVI Committee
	Ongoing - Road warning sign on the fire doors

Ongoing - External lights available for evening use

	6. Fire Exit - Main Hall
	White lines need repainting on external steps
	2
	K Hardy
	Completed Aug 2025

	7. Access to the Clock Tower
	CVI volunteers and employees to only access the clock tower when there is more than one person in the building. CVI Committee and Caretaker to aware of this.
	3
	CVI Committee

K Hardy
	Contractors access the clock tower at their own risk.

	8. First Aider/First Aid Box


	No qualified First Aider on site

Use of personal mobile phones to call 999

Defibulator located outside CVI but information on box is illegible.  

First Aid box is located in kitchen. Poster on notice board (entrance hall), on fuse box in main hall and in hire agreement stating this. Regularly checked and restocked
	2
	B Ward
	Reported to CPC 29/7/25, no action needed as emergency services aware of defib locations. Sign added to say ring 999 
New first aid notice needed (completed Aug 2025)

	9. Building needs to be comfortable for users (too cold)


	Improvements to building in 2024 – under floor insulation, thermal blinds, lower ceiling in back room and insulate, insulated fire door and clock tower and double glazing with comfort plus glass. 

Heating Thermostat Control 

AC / heat system installed in kitchen as no other heating
	2
	CVI Committee


	Building comfortable

	10. Building needs to be comfortable for users (too hot)
	In addition to blind in kitchen AC / heat system installed 

Fan retained in back room to keep building cool in summer

Installed double glazing with comfort plus glass.

Close curtains / blinds and close windows if temperature is hotter outside than inside.
Ad hoc comment that we may need LST radiators in main hall as vulnerable people use it. 
	2
	All
	Building comfortable 
To be discussed at CVIAC 15/7/25, but will need to tie up with future heating needs. Agreed not to change at present but consider as part of building programme in future. 

	11. Electric lighting satisfactory for users
	Improved lighting for Cononley Singers

Did not agree to any further work re lighting
	2
	B Ward
	NFA

	12.  Regular Hirers aware of risk assessments
	Ensure Regular Hirers receive a copy of the CVI General Risk Assessment annually.
Fire risk assessment training 13/1/25


	2
	B Ward
	Annual letter due Sept 2025. No further fire training planned

	13. Food Hygiene
	B Ward has a Basic Food Hygiene Certificate, not required but good health and hygiene is. 

‘Using the kitchen’ instructions on display

Fly light in situ

CVI does not require Food Business Registration. Stepping Stones Nursery responsible for own food hygiene
	2
	CVI Committee
	Ask any other CVI Committee members if they would be willing to complete a Basic Food Hygiene course.



	14. Employers Health and Safety Responsibilities/ Lone Working -

For K Hardy
	Lone working policy in place (reviewed 6/1/25)
	3
	Parish Council
	Ensure Kim Hardy is reminded


	15. General tidiness in the building to prevent accidents and maintain cleanliness
	That all regular hirers keep surfaces clear of obstacles and equipment
	1
	CVI and regular users
	Remind all regular users

	16. Additional furniture
	CVIAC wants to keep the building ‘clutter free’.
	1
	CVI Committee and ALL regular hirers
	Storage is allocated to regular users. No further storage should be used without permission of CVIAC

No additional furniture to be brought into the building with permission of CVIAC

	Main Hall
	Observation
	LEVEL OF RISK
	PERSON(S) RESPONSIBLE
	 ACTION REQUIRED

	17. Tables stacked in Main Hall by chairs
	Tables stored by side of chair rack 
	1
	B Ward and Stepping Stones Nursery
	Check use and agree storage

	18.  Grey Chairs in Main Hall 
	Grey chairs in Main Hall stacked appropriately
	2
	CVI Committee
	Max 5 stacked

	19. Storage cupboards
	Injury risk regarding lifting items from / into cupboards
	3
	B Ward and regular hirers
	Limited heavy items at height. Regular users responsible for own H&S / lifting. Encourage to use ladders

	20. Children’s toys - storage
	Risk of toys falling out of cupboard
	3
	B Ward and Stepping Stones Nursery
	Discussed with Stepping Stones Nursery. Much improved

	21. Chair stand
	Dange of injury from rack
	3
	Andy Gregory
	No cover is available to devise a safety system

	Entrance Hall
	Observation
	LEVEL OF RISK
	PERSON(S) RESPONSIBLE
	 ACTION REQUIRED

	22. Stair lift 
	Risk of sitting on stair lift foot rest rather than seat and slipping onto floor
	2
	B Ward
	Warning sign placed on foot rest (completed September 2025)

	Kitchen
	Observation
	LEVEL OF RISK
	PERSON(S) RESPONSIBLE
	 ACTION REQUIRED

	23. Hatch – Kitchen/Main Hall
	Low level unsighted usage in Main Hall – hazard. Safety sign in kitchen


	2
	CVI Committee
	Ongoing

	24. Dirty mops stored in kitchen
	Possible trip hazards and H&S (cleanliness) issue
	2
	BW, Kim Hardy and Stepping Stones Nursery
	Explore other storage area

Bleach / wash mop heads regularly

	25. Wet floors after cleaning
	Damage to floors

Risk of slipping
	3
	BW, Kim Hardy
	Wipe floors dry after mopping

	26. Stepping Stones calendar above cooker
	Potential fire risk (not paper)
	1
	BW, and Stepping Stones Nursery
	Explore other storage area



	27. Appliances left on - gas hob. Action taken at time – wrote to all relevant regular hirers. Asked to double check. Reminded everyone to double check appliance, even if they have not used it.
	Potential risk of fire 
	3
	CVI Committee, Kim Hardy, ALL regular hirers and private hirers
	Kim to check at each clean

	28. Ad hoc comment that we may need ‘warning hot water signs’
	Possible risk of scalding but children should not be in the kitchen. All other users are independent and aware of hot water taps
	1
	CVIAC
	To put up sign in kitchen. Completed 19/9/25 

	Meeting Room
	Observation
	LEVEL OF RISK
	PERSON(S) RESPONSIBLE
	 ACTION REQUIRED

	29. Fire door into meeting room does not close fully. 
	Potential risk of fire spreading
	4
	BW 
	Replaced August 2025

	Storeroom and Loft
	Observation
	LEVEL OF RISK
	PERSON(S) RESPONSIBLE
	 ACTION REQUIRED

	30. Loft Storage 
	Ensure only light items are stored in the new loft space.

Storage of flammable materials 
Smoke alarm not audible when doors shut
	3
	CVI Committee

K Hardy
	Ensure no significant amount of flammable materials are stored in loft (currently small amount of paint and white spirit)

To discuss need for smoke detector linked to fire alarm CVIAC 15/9/25. To get quotes.

	31.  Chairs in Storeroom 
	Only 1 brown chair in store room for DIY / decorating

Chairs in Store Room stacked appropriately
	1
	CVI Committee
	Mark brown chair as DIY use only. To ask Kim if this is needed

	32. Stepping Stones storage boxes
	Potential risk of falling due to poor organisation. Limited impact as not heavy or public space
	1
	BW, and Stepping Stones Nursery
	Ask Stepping Stones Nursery to re-arrange store room

	33. Cononley Singers keyboard
	Risk of injury when moving
	2
	BW and Cononley Singers
	Maintain clear store room

Cononley Singers responsible for H&S / heavy lifting


LEVELS OF RISK 1 – Very Low 
2 –   Low   3 – Medium  4 –High
 5 – Very high


B Ward and M Allum 29/7/25. Signed off by CVIAC 15/9/25

















Store


Room





Main hall





Meeting Room





Kitchen








10
Risk Assessment Small Premises (FSFRM058A)
Page 9 of 11

